Министерство образования Республики Беларусь

Учреждение образования

БЕЛОРУССКИЙ ГОСУДАРСТВЕННЫЙ УНИВЕРСИТЕТ

ИНФОРМАТИКИ И РАДИОЭЛЕКТРОНИКИ

Факультет компьютерных систем и сетей

Кафедра электронных вычислительных машин

Дисциплина: Автоматизированное проектирование цифровых устройств

ОТЧЕТ ПО ЛАБОРАТОРНОЙ РАБОТЕ №2

на тему

«Использование модулей памяти»

Вариант 4

Выполнил: студент группы 250541

Власов Р. Е.

Проверил: Воронов А.А.

Минск 2026

1 Цель работы

Целью лабораторной работы является разработка и проверка VHDL-модели цифрового устройства, включающего постоянную память ROM, оперативную память RAM с общей шиной данных и управляющий автомат переноса слова из ячейки-источника в ячейку-приемник.

Дополнительно требуется подготовить проект в двух САПР: Intel Quartus II и Xilinx Vivado, получить временные диаграммы, RTL-представление, результаты компиляции и синтеза, а также подтвердить работоспособность автоматическим testbench.

2 Задание и исходные данные

По методическим указаниям необходимо разработать блок, в который входят модули памяти lpm_rom и lpm_ram_io, подключенные к общей восьмиразрядной шине. Требуется продемонстрировать чтение из памяти и выполнить пересылку данных из ROM в RAM через восьмиразрядный регистр.

Таблица 1 – Исходные данные

| Параметр | Значение для варианта 4 |
| --- | --- |
| Синхронность ввода lpm_rom | Асинхронный ввод адреса |
| Синхронность вывода lpm_rom | Синхронный вывод данных |
| Синхронность ввода lpm_ram_io | Синхронная запись данных |
| Синхронность вывода lpm_ram_io | Синхронное чтение данных |
| Ячейка-источник | ROM[4] |
| Ячейка-приемник | RAM[5] |
| Передаваемое значение | 0x5A |

3 Теоретические сведения

Постоянная память ROM хранит заранее заданный набор слов и в данной работе используется как источник данных. Оперативная память RAM допускает запись и последующее чтение по адресу. В FPGA внутренние двунаправленные шины и высокоимпедансные состояния обычно преобразуются синтезатором в логические мультиплексоры, однако в функциональной модели VHDL состояние 'Z' удобно показывает, какой узел в данный момент управляет общей шиной.

Синхронный вывод памяти означает, что новое значение появляется на выходе только после фронта тактового сигнала. Поэтому для варианта 4 чтение ROM и чтение RAM имеют задержку на один такт. Это учитывается управляющим автоматом: после включения ROM выполняется отдельное состояние загрузки регистра, а после записи в RAM выполняется состояние синхронного чтения.

Модель lpm_rom реализована как массив из шестнадцати восьмиразрядных слов. Модель lpm_ram_io содержит массив RAM 16 x 8, вход записи, вход разрешения вывода и общий порт data_io. При записи значение берется с общей шины, а при чтении RAM сама выводит слово на эту шину.

Рисунок 3 - Иерархическое представление разрабатываемого блока

Рисунок 4 - Граф состояний управляющего автомата

4 Разработка VHDL-модели

Главный модуль memory_transfer содержит константы адресов источника и приемника, экземпляры ROM и RAM, регистр данных и конечный автомат. Вариант 4 задан константами SRC_ADDR_C = 4, DST_ADDR_C = 5 и EXPECTED_C = 0x5A.

Автомат имеет шесть состояний: S_IDLE, S_READ_ROM, S_LOAD_REG, S_WRITE_RAM, S_READ_RAM и S_DONE. В S_READ_ROM включается ROM, в S_LOAD_REG регистр захватывает слово 0x5A, в S_WRITE_RAM регистр выдает слово на шину и активируется запись RAM, в S_READ_RAM выполняется синхронное чтение RAM, а в S_DONE формируются done_o и verify_ok_o.

Таблица 2 – Сигналы для разработки и моделирования модуля

| Сигнал | Назначение |
| --- | --- |
| clk_i | Тактовый сигнал всех синхронных элементов. |
| rst_i | Сброс автомата в состояние ожидания и очистка регистра. |
| start_i | Запуск операции переноса. |
| data_bus_o | Наблюдаемая общая шина данных. |
| rom_q_o, reg_q_o, ram_q_o | Контрольные выходы ROM, регистра и RAM для моделирования. |
| done_o | Операция переноса завершена. |
| verify_ok_o | Прочитанное из RAM значение совпадает с ожидаемым 0x5A. |

5 Функциональное моделирование

Testbench tb_memory_transfer формирует тактовый сигнал с периодом 10 ns, снимает reset, подает start и по тактам проверяет состояние автомата, адреса памяти, состояние общей шины, содержимое регистра и флаг verify_ok. Проверки выполнены операторами assert, поэтому ошибка сразу завершила бы моделирование с severity failure.

Рисунок 5 - Покрытие проверок testbench

6 Реализация проекта в Intel Quartus II

Для Quartus подготовлены файлы memory_transfer.qpf и memory_transfer.qsf. Верхним уровнем назначен memory_transfer_rtl_view_top, в котором биты общей шины выведены отдельными портами BUS_D0...BUS_D7 для удобного просмотра RTL-схемы.

В методических указаниях указано семейство Flex10K. Установленная версия Quartus II 9.1 Web Edition не принимает строки FAMILY "FLEX10K" и FAMILY "FLEX10KE", что зафиксировано в логе проверки. Для получения полной компиляции и отчетов проект собран на доступном семействе Stratix II. VHDL-описание при этом не изменялось.

Рисунок 6 - RTL-представление проекта в Quartus II

Рисунок 7 – Структурная схема проекта в Quartus

Quartus успешно выполнил Analysis & Synthesis, Fitter, Assembler и Classic Timing Analyzer. В отчете синтеза RAM распознана как altsyncram, что подтверждает аппаратную интерпретацию массива памяти, а не только поведенческое моделирование.

Рисунок 8 – Временная диаграмма проекта в Quartus

По временной диаграмме видно, что ROM начинает выдавать 0x5A после тактового фронта, затем это значение фиксируется в регистре, после чего регистр управляет общей шиной при записи в RAM[5]. На завершающем этапе RAM синхронно выводит 0x5A, и флаг verify_ok_o становится равным единице.

7 Реализация проекта в Xilinx Vivado

Для Vivado подготовлены create_project.tcl, memory_transfer.xdc и скрипт run_vivado_synth.ps1.

Рисунок 9 - RTL-представление проекта для Vivado

Рисунок 10 - RTL-представление внутри модуля u_memory_transfer

Рисунок 11 - RTL-представление внутри модуля u_ram

Рисунок 12 – Временная диаграмма проекта в Vivado

8 Вывод

В ходе лабораторной работы разработана VHDL-модель блока памяти по варианту 4. Реализованы модели ROM и RAM, общая шина данных, восьмиразрядный регистр и управляющий автомат переноса данных из ROM[4] в RAM[5].

Работоспособность подтверждена автоматическим testbench и временными диаграммами GHDL. Проект успешно собран в Quartus II и синтезирован в Xilinx Vivado. Получены RTL-представления, отчеты ресурсов и временные диаграммы.

Приложение А. Листинги VHDL-модулей

Модель ROM

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

entity lpm_rom_model is

port (

clk_i     : in  std_logic;

address_i : in  std_logic_vector(3 downto 0);

memenab_i : in  std_logic;

q_o       : out std_logic_vector(7 downto 0)

);

end entity;

architecture rtl of lpm_rom_model is

type memory_t is array (0 to 15) of std_logic_vector(7 downto 0);

signal q_reg : std_logic_vector(7 downto 0) := (others => 'Z');

constant ROM_C : memory_t := (

0  => x"11",

1  => x"27",

2  => x"3C",

3  => x"45",

4  => x"5A",

5  => x"6E",

6  => x"73",

7  => x"8F",

8  => x"91",

9  => x"A4",

10 => x"B8",

11 => x"C2",

12 => x"D5",

13 => x"E9",

14 => x"F0",

15 => x"0D"

);

begin

process(clk_i)

begin

if rising_edge(clk_i) then

if memenab_i = '1' then

q_reg <= ROM_C(to_integer(unsigned(address_i)));

else

q_reg <= (others => 'Z');

end if;

end if;

end process;

q_o <= q_reg;

end architecture;

Модель RAM

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

entity lpm_ram_io_model is

port (

clk_i     : in    std_logic;

address_i : in    std_logic_vector(3 downto 0);

we_i      : in    std_logic;

out_en_i  : in    std_logic;

data_io   : inout std_logic_vector(7 downto 0);

q_dbg_o   : out   std_logic_vector(7 downto 0)

);

end entity;

architecture rtl of lpm_ram_io_model is

type memory_t is array (0 to 15) of std_logic_vector(7 downto 0);

signal ram : memory_t := (others => (others => '0'));

signal q_reg : std_logic_vector(7 downto 0) := (others => 'Z');

begin

process(clk_i)

begin

if rising_edge(clk_i) then

if we_i = '1' then

ram(to_integer(unsigned(address_i))) <= data_io;

end if;

if out_en_i = '1' and we_i = '0' then

q_reg <= ram(to_integer(unsigned(address_i)));

else

q_reg <= (others => 'Z');

end if;

end if;

end process;

data_io <= q_reg when out_en_i = '1' and we_i = '0' else (others => 'Z');

q_dbg_o <= q_reg;

end architecture;

Управляющий модуль memory_transfer

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

entity memory_transfer is

port (

clk_i       : in  std_logic;

rst_i       : in  std_logic;

start_i     : in  std_logic;

done_o      : out std_logic;

busy_o      : out std_logic;

verify_ok_o : out std_logic;

state_o     : out std_logic_vector(2 downto 0);

data_bus_o  : out std_logic_vector(7 downto 0);

rom_q_o     : out std_logic_vector(7 downto 0);

reg_q_o     : out std_logic_vector(7 downto 0);

ram_q_o     : out std_logic_vector(7 downto 0);

rom_addr_o  : out std_logic_vector(3 downto 0);

ram_addr_o  : out std_logic_vector(3 downto 0)

);

end entity;

architecture rtl of memory_transfer is

constant SRC_ADDR_C  : std_logic_vector(3 downto 0) := x"4";

constant DST_ADDR_C  : std_logic_vector(3 downto 0) := x"5";

constant EXPECTED_C  : std_logic_vector(7 downto 0) := x"5A";

type state_t is (S_IDLE, S_READ_ROM, S_LOAD_REG, S_WRITE_RAM, S_READ_RAM, S_DONE);

signal state_reg : state_t := S_IDLE;

signal state_next : state_t;

signal data_bus : std_logic_vector(7 downto 0);

signal rom_q : std_logic_vector(7 downto 0);

signal ram_q : std_logic_vector(7 downto 0);

signal reg_q : std_logic_vector(7 downto 0) := (others => '0');

signal rom_en : std_logic;

signal reg_oe : std_logic;

signal ram_we : std_logic;

signal ram_oe : std_logic;

begin

U_ROM : entity work.lpm_rom_model

port map (

clk_i     => clk_i,

address_i => SRC_ADDR_C,

memenab_i => rom_en,

q_o       => rom_q

);

U_RAM : entity work.lpm_ram_io_model

port map (

clk_i     => clk_i,

address_i => DST_ADDR_C,

we_i      => ram_we,

out_en_i  => ram_oe,

data_io   => data_bus,

q_dbg_o   => ram_q

);

data_bus <= rom_q when rom_en = '1' else (others => 'Z');

data_bus <= reg_q when reg_oe = '1' else (others => 'Z');

process(state_reg, start_i)

begin

state_next <= state_reg;

case state_reg is

when S_IDLE =>

if start_i = '1' then

state_next <= S_READ_ROM;

end if;

when S_READ_ROM =>

state_next <= S_LOAD_REG;

when S_LOAD_REG =>

state_next <= S_WRITE_RAM;

when S_WRITE_RAM =>

state_next <= S_READ_RAM;

when S_READ_RAM =>

state_next <= S_DONE;

when S_DONE =>

if start_i = '0' then

state_next <= S_IDLE;

end if;

end case;

end process;

process(clk_i)

begin

if rising_edge(clk_i) then

if rst_i = '1' then

state_reg <= S_IDLE;

reg_q <= (others => '0');

else

if state_reg = S_LOAD_REG then

reg_q <= data_bus;

end if;

state_reg <= state_next;

end if;

end if;

end process;

rom_en <= '1' when state_reg = S_READ_ROM or state_reg = S_LOAD_REG else '0';

reg_oe <= '1' when state_reg = S_WRITE_RAM else '0';

ram_we <= '1' when state_reg = S_WRITE_RAM else '0';

ram_oe <= '1' when state_reg = S_READ_RAM or state_reg = S_DONE else '0';

done_o      <= '1' when state_reg = S_DONE else '0';

busy_o      <= '0' when state_reg = S_IDLE or state_reg = S_DONE else '1';

verify_ok_o <= '1' when state_reg = S_DONE and ram_q = EXPECTED_C else '0';

with state_reg select state_o <=

"000" when S_IDLE,

"001" when S_READ_ROM,

"010" when S_LOAD_REG,

"011" when S_WRITE_RAM,

"100" when S_READ_RAM,

"101" when others;

data_bus_o <= data_bus;

rom_q_o    <= rom_q;

reg_q_o    <= reg_q;

ram_q_o    <= ram_q;

rom_addr_o <= SRC_ADDR_C;

ram_addr_o <= DST_ADDR_C;

end architecture;

Верхний уровень для RTL Viewer

library ieee;

use ieee.std_logic_1164.all;

entity memory_transfer_rtl_view_top is

port (

CLOCK     : in  std_logic;

RST       : in  std_logic;

START     : in  std_logic;

DONE      : out std_logic;

BUSY      : out std_logic;

OK        : out std_logic;

BUS_D0    : out std_logic;

BUS_D1    : out std_logic;

BUS_D2    : out std_logic;

BUS_D3    : out std_logic;

BUS_D4    : out std_logic;

BUS_D5    : out std_logic;

BUS_D6    : out std_logic;

BUS_D7    : out std_logic

);

end entity;

architecture structural of memory_transfer_rtl_view_top is

signal data_bus_s : std_logic_vector(7 downto 0);

signal unused_state_s : std_logic_vector(2 downto 0);

signal unused_rom_q_s : std_logic_vector(7 downto 0);

signal unused_reg_q_s : std_logic_vector(7 downto 0);

signal unused_ram_q_s : std_logic_vector(7 downto 0);

signal unused_rom_addr_s : std_logic_vector(3 downto 0);

signal unused_ram_addr_s : std_logic_vector(3 downto 0);

begin

U_MEMORY_TRANSFER : entity work.memory_transfer

port map (

clk_i       => CLOCK,

rst_i       => RST,

start_i     => START,

done_o      => DONE,

busy_o      => BUSY,

verify_ok_o => OK,

state_o     => unused_state_s,

data_bus_o  => data_bus_s,

rom_q_o     => unused_rom_q_s,

reg_q_o     => unused_reg_q_s,

ram_q_o     => unused_ram_q_s,

rom_addr_o  => unused_rom_addr_s,

ram_addr_o  => unused_ram_addr_s

);

BUS_D0 <= data_bus_s(0);

BUS_D1 <= data_bus_s(1);

BUS_D2 <= data_bus_s(2);

BUS_D3 <= data_bus_s(3);

BUS_D4 <= data_bus_s(4);

BUS_D5 <= data_bus_s(5);

BUS_D6 <= data_bus_s(6);

BUS_D7 <= data_bus_s(7);

end architecture;

Приложение Б. Листинг testbench

tb_memory_transfer

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

entity tb_memory_transfer is

end entity;

architecture sim of tb_memory_transfer is

signal clk       : std_logic := '0';

signal rst       : std_logic := '1';

signal start     : std_logic := '0';

signal done      : std_logic;

signal busy      : std_logic;

signal verify_ok : std_logic;

signal state     : std_logic_vector(2 downto 0);

signal data_bus  : std_logic_vector(7 downto 0);

signal rom_q     : std_logic_vector(7 downto 0);

signal reg_q     : std_logic_vector(7 downto 0);

signal ram_q     : std_logic_vector(7 downto 0);

signal rom_addr  : std_logic_vector(3 downto 0);

signal ram_addr  : std_logic_vector(3 downto 0);

procedure tick(signal clk_s : in std_logic) is

begin

wait until rising_edge(clk_s);

wait for 1 ns;

end procedure;

begin

clk <= not clk after 5 ns;

DUT : entity work.memory_transfer

port map (

clk_i       => clk,

rst_i       => rst,

start_i     => start,

done_o      => done,

busy_o      => busy,

verify_ok_o => verify_ok,

state_o     => state,

data_bus_o  => data_bus,

rom_q_o     => rom_q,

reg_q_o     => reg_q,

ram_q_o     => ram_q,

rom_addr_o  => rom_addr,

ram_addr_o  => ram_addr

);

stimulus : process

begin

tick(clk);

rst <= '0';

start <= '1';

tick(clk);

assert state = "001" report "FSM must enter ROM read state" severity failure;

assert rom_addr = x"4" report "ROM source address must be 4" severity failure;

tick(clk);

assert state = "010" report "FSM must enter register load state" severity failure;

assert data_bus = x"5A" report "Register must see ROM[4] value 0x5A on shared bus" severity failure;

tick(clk);

assert state = "011" report "FSM must enter RAM write state" severity failure;

assert reg_q = x"5A" report "Register must capture value 0x5A" severity failure;

assert ram_addr = x"5" report "RAM destination address must be 5" severity failure;

assert data_bus = x"5A" report "Register must drive value 0x5A during RAM write" severity failure;

tick(clk);

assert state = "100" report "FSM must enter RAM readback state" severity failure;

tick(clk);

assert done = '1' report "Transfer must finish with done=1" severity failure;

assert verify_ok = '1' report "Verification flag must be active after successful transfer" severity failure;

assert ram_q = x"5A" report "RAM[5] must contain transferred value 0x5A" severity failure;

assert data_bus = x"5A" report "RAM readback must drive 0x5A onto the bus" severity failure;

start <= '0';

tick(clk);

assert done = '0' report "FSM must return to idle after start is released" severity failure;

assert false report "tb_memory_transfer: TEST PASSED" severity note;

wait;

end process;

end architecture;

Приложение В. Файлы проектов САПР

Quartus QSF

set_global_assignment -name FAMILY "Stratix II"

set_global_assignment -name DEVICE AUTO

set_global_assignment -name TOP_LEVEL_ENTITY memory_transfer_rtl_view_top

set_global_assignment -name RESERVE_ALL_UNUSED_PINS "AS INPUT TRI-STATED"

set_global_assignment -name VHDL_FILE ../src/lpm_rom_model.vhd

set_global_assignment -name VHDL_FILE ../src/lpm_ram_io_model.vhd

set_global_assignment -name VHDL_FILE ../src/memory_transfer.vhd

set_global_assignment -name VHDL_FILE ../src/memory_transfer_rtl_view_top.vhd

set_global_assignment -name SIMULATION_MODE FUNCTIONAL

set_global_assignment -name VECTOR_OUTPUT_FORMAT VCD

set_global_assignment -name LAST_QUARTUS_VERSION 9.1

set_global_assignment -name PARTITION_NETLIST_TYPE SOURCE -section_id Top

set_global_assignment -name PARTITION_COLOR 16764057 -section_id Top

set_global_assignment -name LL_ROOT_REGION ON -section_id "Root Region"

set_global_assignment -name LL_MEMBER_STATE LOCKED -section_id "Root Region"

set_instance_assignment -name PARTITION_HIERARCHY root_partition -to | -section_id Top

Vivado TCL

set script_dir [file dirname [file normalize [info script]]]

set root_dir [file normalize [file join $script_dir ".."]]

set project_dir [file join $script_dir "memory_transfer_vivado"]

create_project memory_transfer $project_dir -part xc7a35tcpg236-1 -force

set_property target_language VHDL [current_project]

add_files [file join $root_dir "src" "lpm_rom_model.vhd"]

add_files [file join $root_dir "src" "lpm_ram_io_model.vhd"]

add_files [file join $root_dir "src" "memory_transfer.vhd"]

add_files [file join $root_dir "src" "memory_transfer_rtl_view_top.vhd"]

add_files -fileset sim_1 [file join $root_dir "tb" "tb_memory_transfer.vhd"]

add_files -fileset constrs_1 [file join $script_dir "memory_transfer.xdc"]

set_property top memory_transfer_rtl_view_top [current_fileset]

set_property top tb_memory_transfer [get_filesets sim_1]

update_compile_order -fileset sources_1

update_compile_order -fileset sim_1

launch_runs synth_1

wait_on_run synth_1

open_run synth_1 -name synth_1

write_checkpoint -force [file join $project_dir "memory_transfer_synth.dcp"]